Angela's Tools · Chapter 3: Substances and Their Properties · Lab 2 of 3 · Student recording sheet

# Pick the Spoon: Property Data Decides

Investigation question: A kit for melting candle wax and chocolate needs a stirring spoon. Seven materials are available. Which one should the company choose, and how do you decide fairly when no material is perfect?

Name: ____________________________    Group: ________    Date: ____________

## Your role

☐ Data lead: Reads each property from the table aloud, computes spoon mass and cost for each candidate, and checks the arithmetic of the others.

☐ Screener: Applies the three must-pass criteria to each candidate, records pass or fail with the property that caused the fail, and keeps the list of survivors.

☐ Scorer: Scores the survivors on the four scored criteria using the group's agreed scale and totals the matrix.

☐ Station tester: Runs the verification station: measures mass and volume for the four samples, records what the hot-water test does, and reports back whether the table values held up.

## Safety

• The hot-water station is teacher-operated only. Water at 80 °C scalds instantly; students observe from arm's length and never pour, reach into or move the container. Use tongs for every sample.

• Keep the hot-water container on a tray away from the bench edge and away from cords and backpacks.

• Steel bolts dropped into a glass cylinder can crack it; lower them in with the cylinder tilted, or use a plastic cylinder.

• Pine dust and wax shavings can irritate eyes; brush samples off the bench into the waste, not onto the floor.

• No flames anywhere. The flammability column in the table comes from reference information, not from a demonstration.

• Wipe up water spills at once; wet floors near a balance table are a slip hazard.

## Procedure

1. Read the design brief on the recording sheet: the spoon will rest in a pot of melted wax or chocolate at up to 90 °C, be washed in hot soapy water, sit near a stove burner, have a volume of about 30 cm³, and be sold in a low-cost kit. Underline every requirement.

2. Data lead: for each of the seven materials, compute the spoon's mass (density × 30 cm³) and its material cost (mass in kg × cost per kg). Write both in the screening table.

3. Screener: apply the three must-pass criteria to every material. Criterion 1: must not melt or soften below 120 °C. Criterion 2: must not dissolve in hot water or soak up enough water to swell or stay damp (a material that absorbs only a little, like nylon, passes; wood does not). Criterion 3: spoon mass must be 250 g or less. Mark pass or fail for each and write which property caused any fail. A material that fails any one criterion is out, no matter how good it is elsewhere.

4. Scorer: for the survivors only, score four criteria from 3 (best) to 1 (worst): lighter is better, cheaper is better, not burning in a stove flame is better, and a handle that stays cool (does not conduct heat quickly) is better. Total the scores.

5. Station tester (while steps 2 to 4 run): measure the density of the steel bolts, the PP caps and the wax chunk by water displacement in the 100 mL cylinder (push floating samples fully under with the wire; read the rise). Measure the pine block with the ruler instead (length × width × height) because it soaks up water. Weigh each sample and compute density; compare with the table.

6. When called, bring the PP cap and the wax chunk to the teacher's hot-water station. Watch for two minutes as the teacher places both in 80 °C water with tongs. Record what happens to each.

7. As a group, compare the station results with the table. Did the measured densities match within about 0.1 g/cm³? Did the hot-water test agree with the melting or softening column?

8. Decide. Write the group's recommended material, the two strongest reasons from the data, and the trade-off the chosen material still has. Then sketch a two-material spoon that fixes that trade-off, labeling which material goes where and which property of each you are using.

9. Clean the station: dry the samples, return them to the kit, empty and rinse the cylinder, wipe the bench. Answer the analysis questions individually.

## Record your observations

## Lab 2 recording sheet: Pick the spoon

Design brief

A company sells a kit for melting candle wax and chocolate in a pot on the stove. The kit needs a stirring spoon. The spoon rests in melted wax or chocolate at up to 90 °C, is washed afterward in hot soapy water, sits beside a stove burner, has a volume of about 30 cm³, and must keep the kit cheap. Choose the material.

Property data for seven candidate materials (rounded teaching values)

Simulated teaching dataset (not real measurements).

| Material | Density (g/cm³) | Melts or softens at (°C) | Dissolves in or soaks up water? | Burns in a stove flame? | Conducts heat quickly? | Cost ($ per kg) |
| --- | --- | --- | --- | --- | --- | --- |
| Aluminum | 2.70 | 660 | No | No | Yes | 3 |
| Stainless steel | 7.9 | about 1400 | No | No | Yes, but slower than aluminum | 4 |
| Polypropylene (PP) | 0.90 | about 160 | No | Yes | No | 2 |
| PLA plastic | 1.24 | softens about 60 | No | Yes | No | 4 |
| Nylon | 1.14 | about 220 | Soaks up a little | Yes | No | 5 |
| Pine wood | 0.50 | does not melt; chars above about 250 | Soaks up water | Yes | No | 1 |
| Paraffin wax | 0.90 | about 55 | No | Yes | No | 2 |

This is a teaching dataset: densities and temperatures are rounded from reference values, and the costs are rough approximations that change with the market.

Step 1: screen every material (must-pass criteria)

| Material | Spoon mass (g) = density × 30 | Cost per spoon ($) | C1: stays solid above 120 °C? | C2: no dissolving or soaking? | C3: mass 250 g or less? | Still in? (property that failed, if any) |
| --- | --- | --- | --- | --- | --- | --- |
| Aluminum |  |  |  |  |  |  |
| Stainless steel |  |  |  |  |  |  |
| Polypropylene (PP) |  |  |  |  |  |  |
| PLA plastic |  |  |  |  |  |  |
| Nylon |  |  |  |  |  |  |
| Pine wood |  |  |  |  |  |  |
| Paraffin wax |  |  |  |  |  |  |

Step 2: score the survivors (3 = best, 2 = middle, 1 = worst)

| Surviving material | Lighter is better | Cheaper is better | Does not burn is better | Cool handle (poor heat conductor) is better | Total |
| --- | --- | --- | --- | --- | --- |

Verification station: does the table hold up?

| Sample | Mass (g) | Volume method (water rise or ruler) | Volume (cm³) | Measured density (g/cm³) | Table value | Within 0.1? |
| --- | --- | --- | --- | --- | --- | --- |
| Steel bolts |  |  |  |  | 7.9 |  |
| PP caps |  |  |  |  | 0.90 |  |
| Pine block |  |  |  |  | 0.50 |  |
| Wax chunk |  |  |  |  | 0.90 |  |

Hot-water test (teacher station, about 80 °C, two minutes)

| Sample | Before | After 2 minutes | Agrees with the 'melts or softens' column? |
| --- | --- | --- | --- |
| PP cap |  |  |  |
| Wax chunk |  |  |  |

1. Our recommendation: name the material, give the two strongest reasons from the data, and state the trade-off this material still has.

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

2. Which material was eliminated by the screen even though it had the best score on at least two of the scored criteria? Which property eliminated it?

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

Sketch a spoon made of two materials that fixes your recommended material's trade-off. Label which material forms the bowl and which forms the handle, and next to each label write the property from the table that you are using.

## Analysis questions

3. Show the calculation for the stainless steel spoon's mass and its material cost. Does it pass criterion 3?

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

4. Why does the process screen on must-pass criteria before scoring? Give an example from this lab of a material that would have ranked near the top if it had been scored without screening.

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

5. Pine wood is the lightest and cheapest material in the table, yet it was eliminated. Which property eliminated it, and why does that property matter for a spoon that is washed in hot soapy water?

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

6. At the station, which sample's measured density was farthest from the table value? Suggest a reason connected to how you measured its volume.

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

7. In 80 °C water the wax melted but the polypropylene cap did not change. Which property did that test check? Predict, with a reason, what PLA plastic would have done in the same water.

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

8. A classmate argues: 'Stainless steel is best because it has the highest melting point.' Use the full screen-then-score process to explain what this argument gets right and what it leaves out, and say what the two-material spoon in your drawing is trying to fix.

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________

______________________________________________________________________________________